AUTOMOTIVE INDUSTRY STANDARD

UNIFORM PROVISIONS CONCERNING THE APPROVAL OF VEHICLES WITH REGARDS TO

EVENT DATA RECORDER (EDR)

Date of hosting on website: 16th December, 2025

Last date for comments: 14th January, 2026

## INTRODUCTION

The Government of India felt the need for a permanent agency to expedite the publication of standards and development of test facilities in parallel when the work on the preparation of the standards is going on, as the development of improved safety critical parts can be undertaken only after the publication of the standard and commissioning of test facilities. To this end, the erstwhile Ministry of Surface Transport (MOST) has constituted a permanent Automotive Industry Standards Committee (AISC) vide order No. RT-11028/11/97-MVL dated September 15, 1997. The standards prepared by AISC will be approved by the permanent CMVR Technical Standing Committee (CMVR-TSC). After approval, the Automotive Research Association of India, (ARAI), Pune, being the Secretariat of the AIS Committee, will publish this standard. For better dissemination of this information ARAI may publish this document on their Website. The intention of this standard is to establish uniform provisions concerning the approval of motor vehicle of the categories M2, M3, N2 and N3 with regard to their Event Data Recorders (EDR’s) The provisions concern the minimum collection, storage and crash survivability of the motor vehicle crash event data. It does not include specifications for data retrieval tools and methods which are subject to national or regional level requirements. The purpose of these provisions is to ensure that EDRs record, in a readily usable manner, data valuable for effective crash investigations and for analysis of safety equipment performance while limiting, to the greatest extent possible, the recording of data unrelated to the crash. Such crash data will help provide a better understanding of the circumstances in which crashes and injuries occur and will facilitate the development of safer vehicle designs. In this context, crashes should be understood as involving property damage and / or personal harm, including that of vulnerable road users involved. It is understood that, in the current state of technology, the aforementioned objective can be reached only by recording the data in a specified time period based on defined triggers and trigger thresholds. These triggers may, but do not always immediately precede or follow or coincide with the crash. While formation of this standard reference is drawn from UN R 169. Composition of the Panel and Automotive Industry Standards Committee (AISC) responsible for preparation and approval of this standard are given in Annexure 5 & Annexure 6 respectively

CONTENTS

| Clause No. | Details | Page No. |
| --- | --- | --- |
| 1.0 | Scope | 1 / 21 |
| 2.0 | Definitions | 1 / 21 |
| 3.0 | Application for Approval | 4 / 21 |
| 4.0 | Approval | 4 / 21 |
| 5.0 | Specification | 4 / 21 |
| 6.0 | Modification of vehicle model and its variants and extension of approval | 6 / 21 |
| Annexures |  |  |
| Annexure 1 | Reserved | 8 / 21 |
| Annexure 2 | Information document on the type approval of a vehicle model and its variants with regard to its event data recorder | 9 / 21 |
| Annexure 3 | Reserved | 11 / 21 |
| Annexure 4 | Data Elements and formats | 12 / 21 |
| Annexure 5 | Composition of AISC Panel on Event Data Recorder (EDR) | 19 / 21 |
| Annexure 6 | AISC Committee Composition | 21 / 21 |

Uniform provisions concerning the approval of vehicles

with regards to Event Data Recorder (EDR)

| 1.0 | SCOPE |  |
| --- | --- | --- |
| 1.1 | This standard applies to the approval of vehicles of categories M2, M3, N2 and N3 with regard to their Event Data Recorders (EDRs) |  |
| 1.2 | This standard is without prejudice to requirements of national or regional laws related to privacy, data protection and personal data processing. |  |
| 1.3 | The following data elements are excluded from the scope: Vehicle Identification Number (VIN), associated vehicle details, location / positioning data, information on the driver, and date and time of an event. |  |
| 1.4 | If there is no system or sensor designed to provide the trigger indicated in the 5.3.1.3 table of safety systems or the data elements to be recorded and stored under clause 5, in the format (range, resolution, and sample rate) indicated in Annexure 4. “Data Elements and Format” or it is not operational at the time of reaching a specific trigger condition as indicated in 5.3.1 or at the time of recording, this document requires neither recording of such data not fitting or making such systems or sensors operational. However, if the vehicle is fitted with an original equipment manufacturer sensor or system designed to provide the trigger indicated in 5.3.1.3 or the data element in the format specified in Annexure 4. “Data Elements and Format”, then it is mandatory to report the data element in the specified format when the sensor or system is operational. In the case the reason for not being operational at the time of recording is a failure of this system or sensor, this failure state shall be recorded by the EDR as defined in Annexure 4. “Data Elements and Format”. |  |
| 2.0 | DEFINITIONS |  |
|  | For the purpose of this standard: |  |
| 2.1 | “Accelerator pedal position” means the actuation of the device that indicates the driver’s demand for acceleration to the propulsion system given in percentage of measured range of the device. This can also cover one pedal drive that may be demanding negative torque or even service braking in lower ranges. |  |
| 2.2 | “Antilock braking system” means a system which detects wheel slip and automatically modulates the pressure producing the braking forces at the wheel (s) to limit the degree of wheel slip. |  |
| 2.3 | “Antilock brake system status – tractor” indicates the status of the antilock brake system on the vehicle / tractor. |  |
| 2.4 | “Antilock brake system status – trailer” indicates the status of the antilock brake system on trailer(s). |  |
| 2.5 | “Advanced Emergency Braking System” means a system which can automatically detect a potential forward collision and activate the vehicle braking system to decelerate the vehicle with the purpose of avoiding or mitigating a collision. The system may also be referred to as “Automatic Emergency Braking System” |  |
| 2.6 | “Brake Status Parking” indicates the status of the switch that is installed to detect whether or not the parking brake has been applied. |  |
| 2.7 | “Brake Status Service” indicates the status of the switch that is installed in the brake system to detect whether the service brake has been applied. |  |
| 2.8 | “Vehicle Stability Function” means vehicle stability control as defined by IS 11852: 2019 or AIS 150 as amended from time to time. The system may also be referred to as “Electronic Stability Control” |  |
| 2.9 | “Propulsion System Activation Hours” means the accumulated time that the propulsion system is activated including idle state. |  |
| 2.10 | “Propulsion System Torque” means the percentage of peak or reference torque |  |
| 2.11 | “Propulsion System Power” means the instantaneous power provided by the propulsion system. |  |
| 2.12 | “Propulsion System Drive Speed” means the rotational speed of the output shaft of the propulsion system. |  |
| 2.13 | “Event” means a crash or other physical occurrence that causes the trigger threshold to be met or exceeded. |  |
| 2.14 | “Event Data Recorder (EDR)” means a device or function in a vehicle, the purpose of which is to record the vehicle’s dynamic, time – series data during the time period just prior to, during, and after an event (e.g. vehicle speed versus time). |  |
| 2.15 | “Event Data Recording Complete” indicates whether a complete set of data that the event data recording device is designed to capture was successfully recorded by and stored in the device. |  |
| 2.16 | “End of Event Time” means the moment at which the cumulative change in velocity within a 20 ms time period becomes 0.8 km/h or less. |  |
| 2.17 | “EDR unit hardware part number” means the part number for the EDR unit. |  |
| 2.18 | “EDR unit software part number” means the part number / version number for the EDR software. |  |
| 2.19 | “Ignition Cycle Event” means the number (count) of power mode cycles as determined by the EDR ECU at the time when the event occurred since the first use of the EDR. |  |
| 2.20 | “Ignition Cycle Download” means the number (count) power mode cycles as determined by the EDR ECU at the time when the data was downloaded since the first use of the EDR. |  |
| 2.21 | “Lane Departure Warning System Status” indicates the status of the lane departure warning system. |  |
| 2.22 | Maximum Delta V, Longitudinal: means the maximum value of the cumulative change in velocity along the longitudinal axis of the vehicle within a time period of 300 ms after time zero or before the end of evet time plus 30 ms, whichever is shorter. |  |
| 2.23 | “Maximum Delta V, lateral” means the maximum value of the cumulative change in velocity along the lateral axis of the vehicle within a time period of 300ms after time zero or before the end of event time plus 30ms, whichever is shorter. |  |
| 2.24 | Maximum Delta V, resultant” means the time correlated maximum value of the cumulative change in velocity, as reported by the EDR along the vector – added longitudinal and lateral axis. |  |
| 2.25 | “Time for maximum delta V,” means the time from time zero to the point where the maximum value of the cumulative change in velocity, as recorded by the EDR, is found. |  |
| 2.26 | “Retarder torque mode” indicates which function is currently generating, limiting, or controlling the retarder torque. |  |
| 2.27 | “Roll angle” means the angle around which the vehicle rotates about its longitudinal axis. |  |
| 2.28 | “Rollover protection control System” means the stability control of brakes for rollover protection. |  |
| 2.29 | “Roll Rate” means the change of angle per unit time at which the vehicle rotates about its longitudinal axis. |  |
| 2.30 | “Supplemental Resistant System” means a passive safety system as declared by the vehicle manufacturer, supplementing the restraint system as defined by IS 15140, with components such as airbags or seatbelt pre-tensioners. |  |
| 2.31 | “Safety Belt Status” means the feedback from the safety system that the vehicle’ safety belt is fastened or not fastened. |  |
| 2.32 | “Steering Wheel Angle” means the angle of the steering shaft connected to the driver control. |  |
| 2.33 | “System Intervention” means the activation of a system, as defined by the manufacturer. |  |
| 2.34 | “Parking Brake System” means the parking braking system as regulated by AIS 150 or IS 11852: 2019 as amended from time to time |  |
| 2.35 | “Time Zero” is the time reference for the EDR data timestamps of an event. |  |
| 2.36 | “Tyre pressure monitoring system status” means the operating status of the tyre pressure monitoring system. |  |
| 2.37 | “Trigger thresholds” means the appropriate parameter has met the conditions for recording an EDR event. |  |
| 2.38 | “Trigger activated” indicates which trigger was activated to cause the recording of the event. |  |
| 2.39 | “Stability Control System Status – Fully Operational” indicates whether Stability Control is fully operational or whether its functionality is reduced by a permanent or temporary (e.g. low voltage) defect, by intended action (e.g. disabled by a switch or during special diagnostic procedures), not configured or not yet fully initialized (e.g. missing initialization or configuration message). |  |
| 2.40 | “Vehicle Master Control Switch: means the device by which the vehicle’s “on-board electronics system is brought, from being switched off, as in the case where a vehicle is parked without the driver being present, to normal operation mode. |  |
| 2.41 | “Vehicle Speed” means the longitudinal speed of the vehicle that is calculated or estimated from the vehicle Speed Sensor (VSS). |  |
| 2.42 | “Vehicle model and its variants with regards to its Event Data Recorder” means vehicles which do not differ significantly in such essential aspects as: |  |
|  | - The manufacturer’s trade name or mark |  |
|  | - Vehicle features which significantly influences the performances of the EDR; Addition of new trigger(s), new data (elements), or modification in their format, shall not be considered as “significantly” influencing the performance of EDR”. |  |
|  | - The main characteristics and design of the EDR |  |
| 2.43 | “Yaw Control System” means the stability control of wheel brakes for yaw control. |  |
| 2.44 | “Corrective Steering Function (CSF) status” means the operating status of the Corrective Steering Function as defined in AIS 193. |  |
| 2.45 | “Emergency Steering Function (ESF) Status” means the operating status of the Emergency Steering Functions as defined in AIS 193. |  |
| 2.46 | “Automatically Commanded Steering Function (ACSF) status” means the operating status of the Automatically Commanded Steering Function as defined in AIS 193, and of the categories as defined therein. |  |
| 2.47 | “Accident Emergency Call System Status” means the status of a system that is activated either automatically via in vehicle sensors or manually, which carries, by means of public mobile wireless communications network, a set of crash related data and establishes an emergency audio channel between the occupants of the vehicle and an answering point. Faulted status would mean when the system detects it cannot make a call. |  |
| 3.0 | APPLICATION FOR APPROVAL |  |
| 3.1 | The application for approval of a Vehicle model and its variants with regard to its EDR shall be submitted by the vehicle manufacturer or by its authorized representative to the Test Agency. |  |
| 3.2 | It shall be accompanied by the following documentation (a model of the information document is given in Annexure 2). |  |
| 3.2.1 | A description of the Vehicle model and its variants with regard to the items specified in clause 5, below, in particular related to the location of the EDR in the vehicle, the triggering parameters, storing capacity and the resistance to high deceleration and mechanical stress of a severe impact. |  |
| 3.2.2 | The data elements and format stored in the EDR; |  |
| 3.2.3 | Instructions for retrieving the data from the EDR. |  |
| 3.3 | A vehicle representative of the Vehicle model and its variants to be approved shall be submitted to the Test Agency responsible for conducting the approval tests. |  |
| 4.0 | APPROVAL |  |
| 4.1 | If the Vehicle model and its variants submitted for approval pursuant to this standard meets the requirements of clause 5, below approval of that Vehicle model and its variants shall be granted. |  |
| 4.1.1 | Vehicles referred to in clause 4.1, which are not subject to national and regional standards providing for crash tests requirements referred to in clause 5.4.1 and 5.4.2 of, AIS 192 as amended time to time shall be subject to clause 5.4 of this standard. |  |
| 5.0 | SPECIFICATIONS |  |
|  | Requirements for vehicles fitted with an EDR include data elements, data format and data capture. |  |
| 5.1 | Data Elements |  |
| 5.1.1 | Each vehicle fitted with an EDR shall record the data elements specified as mandatory and those required under specified minimum conditions during the interval / time and at the sample rate specified in Annexure 4. |  |
| 5.2 | Data format |  |
| 5.2.1 | Each data element recorded shall be reported as specified in Annexure 4, Table 1 |  |
| 5.3 | Data Capture |  |
|  | The EDR shall capture data which shall be written to a non – volatile memory when any of the triggers in clause 5.3.1 occur. |  |
|  | The EDR shall record the captured data in the vehicle and this data shall remain in the vehicle subject to the provisions of clause 5.3.4, at least until they are retrieved in compliance with national or regional legislation, or they are overwritten in compliance with clause 5.3.4 |  |
|  | The EDR non-volatile memory buffer shall accommodate the data related to at least five different events. |  |
|  | The data elements for every event shall be captured and recorded by the EDR as specified in clause 5.1, in accordance with the following conditions and circumstances; |  |
| 5.3.1 | Conditions for triggering recording of data |  |
|  | As event shall be recorded by the EDR if one of the following threshold values is met or exceeded. Triggers that occur such that an overlap of data between events would result may be excluded. |  |
| 5.3.1.1 | Sudden Deceleration: Vehicle speed changes at a rate higher than 3.25 m/s2 and the change persists beyond that threshold for at least 0.7 seconds. |  |
| 5.3.1.2 | Last Stop: Trigger shall be activated if any of the following applies: |  |
|  | - Vehicle Speed is reported as 0 km/h for 20 s |  |
|  | - While vehicle speed is reported as 0 km/h, and |  |
|  | - Parking brake system is applied, or |  |
|  | - Vehicle master control switch is deactivated |  |
|  | Re-activation of last stop trigger due to threshold criterion (a), shall be disabled if the vehicle speed is not reported as 24 km/h or more for a minimum of 6s. |  |
| 5.3.1.3 | Activation of a safety system is showed in the table below: |  |
|  | System (if fitted) | Trigger |
|  | Supplemental Restraint System | Deployment Command of a Supplement Restraint System |
|  | Antilock Braking System | System Intervention |
|  | Advanced Emergency Braking (including pedestrian / cyclist if equipped) | Emergency Brake Intervention |
|  | Vehicle Stability Function | System Intervention |
| 5.3.2 | Conditions for locking of data |  |
|  | In all the cases with supplemental restraint system activation, the memory for the event shall be locked to prevent any future overwriting of the data by subsequent event. |  |
| 5.3.3 | Conditions for the establishment of time zero. |  |
|  | Time zero is established by the occurrence of any of the above triggers, except for the last stop. |  |
|  | Time zero for the last stop trigger is established when the vehicle reaches an indicated speed of 0 km/h. |  |
| 5.3.4 | Overwriting |  |
| 5.3.4.1 | If an EDR non-volatile memory buffer void of previous event data is not available, the recorded data shall, subject to the provisions of clause 5.3.2, be overwritten by the current event data, on a first in first out basis, or according to the relevant authorities. |  |
| 5.3.4.2 | Furthermore, if an EDR non-volatile memory buffer void of previous event data is not available, data originating from supplemental restraint system events shall always overwrite any other data that is not locked per clause 5.3.2 |  |
| 5.3.5 | Power and Communication Failure |  |
|  | Data recorded in the non -volatile memory is retained after a loss of power. However, data need not be recorded when the power or the communication is lost to the device hosting the EDR or systems providing data. |  |
| 5.4 | Survivability |  |
| 5.4.1 | The data elements listed in Annexure 4 shall be retrievable in the format specified even after an impact. Therefore, event data recorders shall resist internal loads which may occur during a vehicle crash and be mounted in the vehicle in a position of sufficient structural integrity to protect against physical damage due to front and side impacts that would prevent the retrieval of data. To demonstrate these capabilities, Option 1 or Option 2 applies at the choice of the manufacturer. |  |
|  | Option 1: |  |
|  | EDR’s shall withstand mechanical shocks at a severity level as specified in the component test of ANNEXURE-IX-C of the AIS-038 (Rev. 2) as amended time to time. The devices shall be connected to the test fixture only by the intended mountings provided for the purpose of attaching the event data recorders to the vehicle and in an orientation representative of the vehicle installation. |  |
|  | EDR device (s) shall be mounted in the vehicle cab / passenger compartment or in a position of sufficient structural integrity to protect against physical damage (mechanical integrity) that would prevent the retrieval of data at least in front and side impacts of a severity level corresponding to the mechanical shock requirements above. For positions outside the vehicle cab / passenger compartment, the sufficient structural integrity shall be demonstrated to the technical service together with appropriate documentation (e.g. calculations or simulations). |  |
|  | Option 2: |  |
|  | The manufacturer demonstrates that data is retrievable even after an impact of a severity level set by AIS 098 (Annex 1), AIS 099 (Annex 1) or AIS 201 (Annex 1). |  |
| 5.5 | It shall not be possible to deactivate the Event Data Recorder. |  |
| 6.0 | MODIFICATION OF VEHICLE MODEL AND ITS VARIANTS AND EXTENSION OF APPROVAL |  |
| 6.1 | Every modification of the Vehicle model and its variants as defined in clause 2.42 of this standard shall be notified to the test agency which approved the Vehicle model and its variants. The test agency may then either: |  |
| 6.1.1 | Consider that the modifications made do not have an adverse effect on the conditions of the granting of the approval and grant an extension of approval; |  |
| 6.1.2 | Consider that the modifications made affect the conditions of the granting of the approval, and require further tests or additional checks before granting an extension of an approval. |  |
| 6.2 | Confirmation or refusal of approval, specifying the alterations, shall be communicated by the procedure specified in clause 4.3 above to the Test Agency of the agreement applying this standard. |  |

ANNEXURE I

RESERVED

| ANNEXURE 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| INFORMATION DOCUMENT ON THE TYPE APPROVAL OF A VEHICLE MODEL AND ITS VARIANTS WITH REGARD TO ITS EVENT DATA RECORDER |  |  |  |  |  |  |
|  | A list of contents shall be included |  |  |  |  |  |
|  | Any drawings shall be supplied in an appropriate scale and in sufficient detail on size A4 paper or on a folder of A4 format |  |  |  |  |  |
|  | Photographs, if any, shall show sufficient detail |  |  |  |  |  |
|  | General |  |  |  |  |  |
| 1.0 | Trade name or mark of vehicle |  |  |  |  |  |
| 2.0 | Vehicle model and its variants |  |  |  |  |  |
| 3.0 | Means of identification of type, if marked on the vehicle |  |  |  |  |  |
| 4.0 | Location of the marking |  |  |  |  |  |
| 5.0 | Location of and method of affixing the approval mark |  |  |  |  |  |
| 6.0 | Category of vehicle |  |  |  |  |  |
| 7.0 | Name and address of manufacturer |  |  |  |  |  |
| 8.0 | Address (es) of assembly plant (s) |  |  |  |  |  |
| 9.0 | Photograph (s) and / or drawing (s) of a representative vehicle |  |  |  |  |  |
| 10.0 | EDR |  |  |  |  |  |
| 10.1 | Make (trade name of manufacturer) |  |  |  |  |  |
| 10.2 | Type and general commercial description (s) |  |  |  |  |  |
| 10.3 | Drawing(s) or photographs showing the location and method of attachment of the EDR in the vehicle |  |  |  |  |  |
| 10.4 | Description of the triggering parameter |  |  |  |  |  |
| 10.5 | Description of any other relevant parameter (Storing capacity, resistance to high declaration and mechanical stress of a severe impact, etc.) |  |  |  |  |  |
| 10.6 | The data elements and data format stored in the EDR. |  |  |  |  |  |
|  | Data elements | Recording interval / time (relative to trigger event) | Data sample rate (samples per second) | Minimum range | Accuracy | Resolution |
| 10.7 | Instructions for retrieving the data from the EDR |  |  |  |  |  |

ANNEXURE 3 RESERVED

| ANNEXURE 4 - DATA ELEMENTS AND FORMAT The data elements specified in Table 1 shall be recorded by the Event Data Recorder if mandated under CMVR Refer clause 1.4 of scope |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 1 – List of Data Elements 1 |  |  |  |  |  |  |  |
| Data element | Condition for requirement (2) | Recording interval / time (3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| Event Data Recording Complete | Mandatory | Following other data | N/A | Yes or No | N/A | Yes or No | All 5.3.1 triggers |
| Propulsion system activation hours event | Mandatory5 | -1.0 sec | N/A | 0 to 1,193,046 hr | ±0.05 hr | 0.05 hr | All 5.3.1 triggers |
| Propulsion system activation hours download | Mandatory6 | At time of download | N/A | 0 to 1,193,046 hr | ± 0.05 hr | 0.05hr | All 5.3.1 triggers |
| EDR unit hardware part number | Mandatory7 | N/A | N/A | N/A | N/A | N/A | N/A |
| EDR unit software part number | Mandatory 7 | N/A | N/A | N/A | N/A | N/A | N/A |
| 1 Format requirements specified below are minimum requirements and manufacturers can exceed them. |  |  |  |  |  |  |  |
| 2. “Mandatory” is subject to the conditions detailed in clause 1 |  |  |  |  |  |  |  |
| 3. Pre-Crash data and crash data are asynchronous. The sample time accuracy requirement for pre-crash time is -0.1 to 1.0 sec (e.g. T =-1 would need to occur between -1.1 and 0 seconds) |  |  |  |  |  |  |  |
| 4. Accuracy requirement only applies within the range of the physical sensor, if measurements captured by a sensor exceed the design range of the sensor, the reported element shall indicate when the measurement first exceeded the design range of the sensor. |  |  |  |  |  |  |  |
| 5. Manufacturers shall record either Propulsion system activation hours event or ignition cycle, event. |  |  |  |  |  |  |  |
| 6. Manufacturers shall record either propulsion system activation hours download or ignition cycle, download. |  |  |  |  |  |  |  |
| 7. Shall not contain unique serial numbers or other unique identifiers. If serial number traceability is integral to part number – it does not need to be reported. |  |  |  |  |  |  |  |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ignition cycle, event | Mandatory5 | -1.0 sec | N/A | 0 to 60,000 | ± 1 cycle | 1 cycle | All 5.3.1 triggers |
| Ignition cycle, download | Mandatory6 | At time of download | N/A | 0 to 60,000 | ± 1 cycle | 1 cycle | All 5.3.1 triggers |
| Vehicle Speed | Mandatory | -20 to +10 sec | 4 | 0 to 250 km/h | ±1 km/h | 1 km/h | All 3.5.1 triggers |
| Gear Position Status | Mandatory9 | -20 to +10 sec | 4 | N/A | N/A | Reverse | All 3.5.1 triggers |
| Retarder Torque Mode | Mandatory / if fitted* | -20 to +10 sec | 4 | N/A | N/A | Default, Operator Selection, Cruise Control, Road Speed Limit, Stability Control, Transmission Control, Engine Speed Limit, Braking System | All 5.3.1 triggers |
| Brake Status – Parking | Mandatory | -20 to +10 sec | 4 | N/A | N/A | On or Off | All 5.3.1 triggers |
| Brake Status Service | Mandatory | -20 to +10 sec | 10 | N/A | N/A | On or Off | All 5.3.1 triggers |
| 8.Event indicates recording around the time of the trigger. |  |  |  |  |  |  |  |
| 9.Gear position is mandatory if the vehicle speed data element only records forward speeds. |  |  |  |  |  |  |  |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Propulsion system Torque10 | Mandatory | -20 to +10 sec | 4 | Actual value reported in Nm | <100Nm ±10 Nm and >100 Nm: 5% of torques value which was recorded | 1Nm | All 5.3.1 triggers |
| Propulsion system power10 | Mandatory | -20 to +10 sec | 4 | Actual value reported in kW | ±5% | 1kW | All 5.3.1 triggers |
| Propulsion system drive speed10 | Mandatory | -20 to +10 sec | 4 | Actual value reported in rpm | ±100 rpm | 100 rpm | All 5.3.1 triggers |
| Accelerator Pedal Position | Mandatory | -20 to +10 sec | 10 | 0 to 100% | ± 5 | 1% | All 5.3.1 triggers |
| ABS Brake Control Status – Motor vehicle | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | NA | Off, ABS Passive but installed, ABS Active | All 5.3.1 triggers |
| ABS Brake Control Status – Trailer | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Off, Faulted, On not intervening On intervening | All 5.3.1 triggers |
| Advanced Emergency Braking | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Off, Faulted, On not warning / inter vanning , On warning, On intervening | All 5.3.1 triggers |
| Lane Departure Warning System Status | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On not warning, On – Warning | All 5.3.1 triggers |
| Steering wheel angle | Mandatory if fitted* | -20 to 10 sec | 10 | -1776 degrees to +1776 degrees | ± 0.4 rad 22.9 degrees | 0.2 rad 11.5 degrees | All 5.3.1 triggers |
| Stability Control System Status | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Fully operational Not fully operational | All 5.3.1 triggers. |
| 10. if motors or other drivers are available as separate items then these should be listed with relevant location, e.g. 1st left or 2nd left, 1st right or 2nd right (driven axles), 1st or 2nd, nth (for combined drivers) for speed, torque and power. For hybrid systems, engine and motor elements should be listed separately. |  |  |  |  |  |  |  |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rollover Protection Control System Status | Mandatory if fitted* | -20 to +10 sec | 4 | N/A | N/A | Passive but installed Active | All 5.3.1 triggers |
| Yaw Control system status | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Passive but installed, Active | All 5.3.1 triggers |
| Safety belt status (position x-y)11 | Mandatory if fitted* | -1.0 sec | N/A | N/A | N/A | Fastened, not fastened | All 5.3.1 triggers |
| Safety belt pre-tensioner status (position x-y)11 | Mandatory if fitted* | Event8 | N/A | N/A | N/A | Faulted, not deployed, deployed | Supplemental restraint system |
| Frontal Airbag system status 11 | Mandatory if fitted* | Event8 | N/A | N/A | N/A | Faulted, suppressed (passenger), deployed not deployed | Supplemental restraint system |
| Side Airbag system status 11 | Mandatory if fitted* | Event 8 | N/A | N/A | N/A | Faulted deployed, not deployed | Supplemental restraint system |
| Side curtain / Tube air bag system status 12 | Mandatory if fitted* | Event 8 | N/A | N/A | N/A | Faulted, deployed, not deployed | Supplemental restraint system |
| Far-side impact centre air bag system status 12 | Mandatory if fitted* | Event 8 | N/A | N/A | N/A | Faulted, deployed, not deployed | Supplemental restraint system |
| Blind Spot Information System for the detection of bicycles | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Off, Faulted, On not warning, On warning left-side. On warning right side. | All 5.3.1 triggers |
| Reversing motion VRU detection system | Mandatory if fitted* | -20 to +10sec | 10 | N/A | N/A | Off, Faulted On not warning, On warning | All 5.3.1 triggers |
| 11. this data element shall be recorded for all seating positions equipped with systems and sensors in accordance with clause 1.4 A separate row shall be added to the report for each seating position, and the position shall be denoted as ‘x=seat row number, starting with 1 at the vehicle front, y= seat number, starting with 1 at the vehicle’s left side. For example, ‘position 1-1 denotes the front leftmost seat and position 1-2 denotes the front second from the left. |  |  |  |  |  |  |  |
| 12. list this element n times, once for each airbag. |  |  |  |  |  |  |  |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Moving Off information system for the Detection of Pedestrian and cyclists | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Off, Faulted, On not warning, On warning | All 5.3.1 triggers |
| Maximum delta – V, longitudinal | Mandatory if fitted* | Event 8 | N/A | -100 km/h to +100 km/h | ± 10% | 1km/h | Supplemental restraint system (planar events) |
| Time, maximum delta- V, longitudinal | Mandatory if fitted* | Event 8 | N/A | 0-300 ms | ± 3ms | 2.5ms | Supplemental restraint system (planar events) |
| Maximum delta- V, Lateral | Mandatory if fitted* | Event 8 | N/A | -100 km/h to +100 km/h | ± 10% | 1km/h | Supplemental restraint system (planar events) |
| Time for maximum delta-V, lateral | Mandatory if fitted* | Event 8 | N/A | 0-300 ms | ±3ms | 2.5 ms | Supplemental restirant system (plannar events) |
| Maximum delta-V resultant | Mandatory if fitted* | Event 8 | N/A | -100 km/h to +100 km/h | ± 10% | 1 km/h | Supplemental restraint system (planar events) |
| Time for maximum delta – V, resultant | Mandatory if fitted* | Event 8 | N/A | 0-300 ms | ± 3ms | 2.5 ms | Supplemental restraint system (planar events) |
| Roll angle | Mandatory if fitted* | -20 to +10 sec | 4 | -1080 deg to +1080 deg | ± 10% | 10 deg | Supplemental restraint system (rollover events) |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Roll rate | Mandatory if fitted and used for rollover occupant protection system control algorithm | -20 to +10 sec | 4 | -240 to +240 deg/sec | ± 10% | 4 deg/sec | Supplemental restraint system (rollover events) |
| Longitudinal Acceleration | Mandatory if fitted* | -20 to +10 sec | 4 | -1.5g to +1.5g | ±10% | 0.1g | All 5.3.1 triggers |
| Lateral acceleration | Mandatory if fitted* | -20 to +10 sec | 4 | -1.5g to +1.5g | ± 10% | 0.1g | All 5.3.1 triggers |
| Accident emergency call system status | Mandatory if fitted* | Event 8 | N/A | N/A | N/A | Faulted, On but emergency call not automatically triggered, On- Emergency call automatically triggered | Supplemental restraint system |
| Tyre pressure monitoring system warning lamp status | Mandatory if fitted* | -20 to +10 sec | 4 | N/A | N/A | On, Off | All 5.3.1 triggers |
| Yaw Rate | Mandatory if fitted* | -20 to +10 sec | 4 | -75 to +75 degrees / second | ± 10% of the full range of the sensor | 1 degree per second | Supplemental restraint system (planar events) |
| Corrective steering function status13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not intervening On actively intervening | All 5.3.1 triggers |
| Emergency steering function status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not intervening, On actively intervening | All 5.3.1 triggers |
| 13. Only applies to vehicles subject to approval in accordance with any UN Regulation annexed to the 1958 agreement with respect to their steering functions. |  |  |  |  |  |  |  |

| Data element | Condition for requirement(2) | Recording interval / time(3) (relative to time zero) | Data sample rate (sample per second) | Minimum range | Accuracy (4) | Resolution | Data recorded for the following triggers |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Automatically commanded steering function category A status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not controlling, On controlling | All 5.3.1 triggers |
| Automatically commanded steering function category B status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not controlling, On controlling | All 5.3.1 triggers |
| Automatically commanded steering function category C status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not controlling, On controlling | All 5.3.1 triggers |
| Automatically commanded steering function category D status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not controlling, On controlling | All 5.3.1 triggers |
| Automatically commanded steering function category E status 13 | Mandatory if fitted* | -20 to +10 sec | 10 | N/A | N/A | Faulted, Off, On but not controlling, On controlling | All 5.3.1 triggers |
| * Note: Mandatory provisioning of this data element shall be in accordance with the notified provisions under CMV(A) R, 1989. As and when such provisions are made mandatory under CMVR, by the Statutory Authority, the data elements for such respective provisions shall be provided by the Vehicle manufacturer and shall be captured by EDR. |  |  |  |  |  |  |  |

| ANNEXURE 5 (See Introduction) COMPOSITION OF AISC PANEL ON EVENT DATA RECORDER (EDR) |  |
| --- | --- |
| Convener |  |
| Shri A. A. Badusha | The Automotive Research Association of India |
| MEMBERS | REPRESENTING |
| Shri Manoj Desai | The Automotive Research Association of India |
| Shri Kamalesh Patil | The Automotive Research Association of India |
| Shri N. Ramesh | The Automotive Research Association of India |
| Shri Pranab Devrajan | The Automotive Research Association of India |
| Shri Vignesh Kumar | The Automotive Research Association of India |
| Shri Pratik Nayak | The Automotive Research Association of India |
| Shri S. Perumal | Global Automotive Research Centre |
| Shri Saurabh | National Automotive Test Tracks |
| Shri N. Muthukumar | Ashok Leyland Ltd., (SIAM) |
| Shri V. Faustino | Ashok Leyland Ltd., (SIAM) |
| Shri V. G. Kulkarni | Mahindra & Mahindra Ltd. (SIAM) |
| Shri Akshay A. P | Mahindra & Mahindra Ltd. (SIAM) |
| Shri Uday Salunkhe | Tata Motors Ltd. (SIAM) |
| Shri Ajay Pandey | Tata Motors Ltd. (SIAM) |
| Shri Rama Manikandan | DICV (SIAM) |
| Shri Venkatesh G | DICV (SIAM) |
| Shri Jebin Jowar | RNTBCI (SIAM) |
| Shri Nithin Roy | Volvo Trucks (SIAM) |
| Shri Ganesh L P | Bosch (ACMA) |
| Shri Vinayak Patil | Bosch (ACMA) |
| Shri Praveen Chilshetty | Knorr Bremse (ACMA) |
| Shri Ruban Devaprasath | Brakes India Ltd (ACMA) |
| Shri G. Devendran | Brakes India Ltd (ACMA) |

| ANNEXURE 6 (See Introduction) COMMITTEE COMPOSITION * Automotive Industry Standards Committee |  |
| --- | --- |
| Chairperson |  |
| Dr. Reji Mathai | Director, The Automotive Research Association of India |
| Members | Representing |
| Representative from | Ministry of Road Transport and Highways |
| Representative from | Ministry of Heavy Industries |
| Representative from | Office of the Development Commissioner, MSME, Ministry of Micro, Small and Medium Enterprises |
| Shri Shrikant R. Marathe | Former Chairman, AISC |
| Head-TED | Bureau of Indian Standards |
| Director | Central Institute of Road Transport |
| Director | Global Automotive Research Centre |
| Director | International Centre for Automotive Technology |
| Director | Indian Institute of Petroleum |
| Director | Vehicles Research and Development Establishment |
| Director | Indian Rubber Manufacturers Research Association |
| Representatives from | Society of Indian Automobile Manufacturers |
| Representatives from | Tractor and Mechanization Association |
| Representatives from | Automotive Components Manufacturers Association of India |
| Representative from | Indian Construction Equipment Manufactures' Association |
| Member Secretary |  |
| Shri Vikram Tandon | The Automotive Research Association of India |

*   At the time of approval of this Automotive Industry Standard (AIS)